Instructions for Running BIB-Briefcase-29

A folder named as BIB_BRIEFCASE_29 will be provided which will contain script for collecting data and pushing data to PI. Few pre-requisite are required before starting the script from the corresponding folder.

The router should have working internet connection. A LAN cable should be required which needs to be plugged at Internet port at the back of router which will provide internet connection to router. The system where the script would be running will be connected to router through another LAN cable plugged at any of 4 Ethernet ports available at router back side. Next, the IP address for system needs to be assigned static IP. The settings to be configured for IP address are as below:

IP address: 172.128.156.252

Subnet Mask: 255.255.0.0

Default Gateway: 172.128.156.245

Preferred DNS: 172.128.156.245

After configuring the settings, run the ‘run’ batch script in dist folder located in BIB_BRIEFCASE_29 folder. After, the script starts to collect the data from the sensors, the decoded data will be saved in corresponding csv files. The csv data files will be stored in ‘C:/BIB29/data’ folder where the data will be in corresponding date folder.

The details of the data format in the csv is explained below:

The data in the csv will be in the following format. One example is explained below:

- SPF_environment_sensor,BIB_BRIEFCASE_29,00:06:66:73:32:bc,1491285106,"[0, 1, 2, 3, 4, 5, 6]","[24.3861, 61.886353, 98.05195, 3.190497E-4, -0.5, 0.671875, -0.515625]"

- SPF_environment_sensor,BIB_BRIEFCASE_29,00:06:66:73:32:bc,1491285106,[11],[1.0]

- SPF_environment_sensor,BIB_BRIEFCASE_29,00:06:66:73:32:bc,1491285106,[12],[1.0]

For Packet 1 field 1 (SPF_environment_sensor) represent device type, field 2 (BIB_BRIEFCASE_29) represents briefcase name, field 3 (00:06:66:73:32:bc) represents MAC address of the sensor, field 4 (1491285106) represents timestamp of the packet, field 5 ([0, 1, 2, 3, 4, 5, 6])represents sensors type starting from temperature [0]; humidity [1]; occupancy (2); light intensity [3]; x-acceleration [4]; y-acceleration [5]; z-acceleration [6], field 6 ([24.3861, 61.886353, 98.05195, 3.190497E-4, -0.5, 0.671875, -0.515625]) represents the value for the above sensors in the same order.

Note: after accessing the above parameters, the light parameters needs to be multiplied by 1000*1000, so for above example the lux value would be 3.190497E-4*1000*1000 = 319.04 lux

Similarly for packet 2, field 5 [11] that’s represents sensor is occupancy state [11] and the corresponding data in field 6 [1.0] represents occupied and vacant states. Occupied state is represented by 1 in field 6 and vacant state is represented by 0.

Similarly, for packet 3, field 5 represents sensor orientation [12] and the field parameters will contain parameters (1, 2, 4, 8, 16, and 32) equivalent to positions (A, B, C, D, E and F) for sensors.

Currently, the sampling time for sensor data is 30 seconds and reporting window is 2 mins.